Communications Test Plan

Numbers refer to Full Test Plan

Monday Lab (TWet_Unl, RWet_Unl, Blend, React_1, Ion_Exc_1 stub, CIP

Will need the following units operational from the Tuesday lab: none

A. Test of RWet_Unl and TWet_Unl with Blend

Start with:

TWet_Unl Shutdown

RWet_Unl Shutdown

Set LT2110 to 0 (from Blend)

Set LT2130 to 0 (from Blend)

From TWet_Unl,

Initiate transfer to T-2110

Set FT1148 to 200

From RWet_Unl

Initiate transfer to T-2110 – should not actually happen

Set LT2110 to 96 (from Blend)

Wait for TWet_Unl to shut down

From RWet_Unl

Initiate transfer to T-2130

Set FT1198 to 200

From TWet_Unl,

Initiate transfer to T-2130 – should not actually happen

Set LT2130 to 96 (from Blend)

Wait for RWet_Unl to shut down

B. Test of CIP with Soda_Ash (should have been done last week)

Start with:

Soda_Ash Shutdown

CIP Shutdown

Run Test 12e

C. Test of React_1 with Blend, QA_Smpl, Ion_Exc_1

Start with:

React_1 Shutdown

Blend shutdown

QA_Smpl Shutdown

Ion_Exc_1 Online

Run Test 15b

D. Test of React_1 with CIP

Start with:

React_1 Shutdown

Blend shutdown

CIP Online/Recycle

Run Test 26e

E. Test of Ion_Exc_1 with CIP

Start with:

Ion_Exc_1 Shutdown

CIP Online/Recycle

Run Test 26c

Tuesday lab (TDry_Unl, RDry_Unl, Dry_Store, React_2, Ion_Exc_2, Stor_Tnk_LO_1)

Will need the following units operational from the Monday lab: Blend, QASmpl, Ion_Exc_1 stub, CIP

F. Test of RDry_Unl and TDry_Unl with Dry_Store

Start with:

TDry_Unl Shutdown

RDry_Unl Shutdown

Set LT2210 to 0 (from Dry_Store)

Set LT2230 to 0 (from Dry_Store)

From TDry_Unl,

Initiate transfer to T-2210

From RDry_Unl

Initiate transfer to T-2210 – should not actually happen

Set LT2210 to 96 (from Dry_Store)

Wait for TDry_Unl to shut down

From RDry_Unl

Initiate transfer to T-2230

From TDry_Unl,

Initiate transfer to T-2230 – should not actually happen

Set LT2230 to 96 (from Dry_Store)

Wait for RDry_Unl to shut down

G. Test of React_2 with Dry_StoOt, Blend, QA_Smpl, Ion_Exc_2

Start with:

React_2 Shutdown

Dry_StoOt Shutdown

Blend shutdown

QA_Smpl Shutdown

Ion_Exc_2 Online

Run Test 16b

H. Test of React_2 with CIP

Start with:

React_2 Shutdown

Blend shutdown

CIP Online/Recycle

Run Test 26f

I. Test of Ion_Exc_2 with CIP

Start with:

Ion_Exc_2 Shutdown

CIP Online/Recycle

Run Test 26d

J. Test of Ion_Exc_2 with Stor_Tnk_1

Start with:

Ion_Exc_2 Shutdown

LT5000 to 40 (from Stor_Tnk_1)

Run Test 19b until Online is initiated

Set LT5000 to 96 (from Stor_Tnk_1)

Ion_Exc_2 should EShutdown

K. Test of Stor_Tnk_1 with CIP

Start with:

Stor_Tnk_1 Shutdown

CIP Online/Recycle

Run Test 26b

Distance Students (Dry_Add) - [not done 2015]

Will need the following units operational from the other labs: Blend, Wet_Add (stub)

L. Test of Dry_Add with Blend

Check that T2150 and T2160 levels can be read from Blend:

Change levels in Blend

M. Test of Dry_Add with Wet_Add

Start with:

Dry_Add Shutdown

Set LT2150 to 0 (from Blend)

Set LT2160 to 0 (from Blend)

At Wet_Add stub

Set Wet_Add_Export_Info to 1

From Dry_Add

Initiate Makeup with destination T-2150 – should not actually happen

At Wet_Add stub

Set Wet_Add_Export_Info to 2

From Dry_Add

Initiate Makeup with destination T-2150 – should progress normall

Wednesday lab (RPow_Unl, React_3, Ion_Exc_3, Ion_Exc_4)

Will need the following units operational from the Distance lab: TPow_Unl, Stor_Tnk_LO_2

N. Test of RPow_Unl with React_3

Start with:

TPow_Unl Shutdown

RPow_Unl Shutdown

Set LT2610 to 0 (from React_3)

Set LT2630 to 0 (from React_3)

From TPow_Unl,

Initiate transfer to T-2610

From RPow_Unl

Initiate transfer to T-2610 – should not actually happen

Set LT2610 to 96 (from React_3)

Wait for TPow_Unl to shut down

From RPow_Unl

Initiate transfer to T-2630

Set LT2630 to 96 (from React_3)

Wait for RPow_Unl to shut down

O. Test of React_3 with Ion_Exc_3

Start with:

React_3 Shutdown

Ion_Exc_3 Online

Run Test 17b

P. Test of React_3 with Ion_Exc_3 and Ion_Exc_4

Start with:

React_3 Shutdown

Ion_Exc_3 Shutdown

Ion_Exc_4 Shutdown

Operator initiate React_3_Flush

Ion_Exc_3 should be commanded to startup and React_3 Flush should progress when it is online

Eventually, React_3 goes to Shutdown (by auto-start)

Q. Test of Ion_Exc_3 with Stor_Tnk_2

Start with:

Ion_Exc_3 Shutdown

LT5500 to 40 (from Stor_Tnk_LO_2)

Run Test 20b until Online is initiated

Set LT5500 to 96 (from Stor_Tnk_LO_2)

Ion_Exc_3 should EShutdown

R. Test of Ion_Exc_4 with Stor_Tnk_2

Start with:

Ion_Exc_4 Shutdown

LT5500 to 40 (from Stor_Tnk_LO_2)

Run Test 21b until Online is initiated

Set LT5500 to 96 (from Stor_Tnk_LO_2)

Ion_Exc_4 should EShutdown

Distance lab (TPow_Unl, Stor_Tnk_LO_2)

Will need the following units operational from the Wednesday lab: RPow_Unl, React_3

S. Test of TPow_Unl with React_3

Start with:

TPow_Unl Shutdown

RPow_Unl Shutdown

Set LT2610 to 0 (from React_3)

Set LT2630 to 0 (from React_3)

From RPow_Unl,

Initiate transfer to T-2610

From TPow_Unl

Initiate transfer to T-2610 – should not actually happen

Set LT2610 to 96 (from React_3)

Wait for RPow_Unl to shut down

From TPow_Unl

Initiate transfer to T-2630

Set LT2630 to 96 (from React_3)

Wait for RPow_Unl to shut down